Experiment # 2

| Lab Title: | Understanding MIPS memory reference and decision making instruction. |  |
| --- | --- | --- |
| Learning Objectives: | To use PCSpim simulator for understanding MIPS memory reference and decision making instructions. |  |
| Tools Used: | PCSpim |  |
| Procedure: |  |  |
| Memory Reference Instructions: Memory reference instructions covered in this lab are described below. Load: lw $s1,100($s2) //load word lh $s1,100($s2) //load half signed lhu $s1,100($s2) //load half unsigned lhui $s1,100 //load upper immediate lb $s1,100($s2) //load byte signed lbu $s1,100($s2) //load byte unsigned Store: sw $s1,100($s2) //stire word sh $s1,100($s2) //stire half signed shu $s1,100($s2) //stire half unsigned sb $s1,100($s2) //stire byte signed sbu $s1,100($s2) //stire byte unsigned Decision making instruction covered in this lab include J 2500 //jump at 10000 beq $s1,$s2,25 //jump on equal bne $s1,$s2,25 //jump on not equal Practice: Make an assembly file (let load_store.asm) and write following program in it. //Place 0x27959381 into $24 lui $24,0x2795 ori $24,$24,0x9381 //$24 into address 0x10000000 lui $8,0x1000 sw $24,0($8) //Load word, half ord and bytes into different registers lw $9,0($8) lbu $10,0($8) lb $11,0($8) lhu $12,0($8) lh $13,0($8) Now take hex code of lw $9,0($8) from PCSpim and convert it into binary(32 bit) using calculator. Separate this binary(append zeros to leftside if this binay is not 32 bit) according to the following format and get decimal values after seperation. Load_Store (I format) opcode; Rs; rt; Offset. 6 bits; 5 bits (base register); 5 bits (destination register for load, source register for store); 16 bits (offset). |
| Exercises: |  |  |
| Complete following lab tasks. 1 Write and run MIPS code of A[0] = 4 A[1]= 0x27959381; B[0] = 3 B[1]= 5 ; C[0] = A[0] + B[0] ; C[1] = A[1] + B[1] ; Where A , B ,C are Arrays of Words. (for Help see Load_Store.asm file you saved previously in practice session) - Following is a C language code if(a = = b) c = d – e ; else c = d + e ; Following is it’s MIPS equavilent assembly program. addi $16,$0,6 #a=6 ori $17,$0,6 #b=6 addi $18,$0,9 #initialization c=9 addi $19,$0,5 #d=5 ori $20,$0,2 #e=2 bne $16,$17,ELSE sll $0,$0,0 # nop no operation to take carry of delayed branch sub $18,$19,$20 # sub if equal(a==b) j EXIT sll $0,$0,0 # nop no operation to take care of delayed branch ELSE: add $18,$19,$20 # else (a!=b) EXIT: Write this code in an assembly file and load it into PCSpim and run the code. Now convert the hex xode of bne and j instructions into binary and separate different fields using following Formats. bne (I format ) opcode; Rs; rt; offset. 6 bits; 5 bits; 5 bits; 16 bits. What is the address of the target instruction ( add $18,$19,$20 instruction) Target address of bne instruction is calculated as PC + 4 + ( offset x 4 ). Compare address of the target instruction and your calculated address from the binary code of branch instruction. J (J format ) # Opcode; Offset. 6 bits; 26 bits. Target address of J instruction is calculated as Target address = [ Left most 4 bits of PC , offset 26 bits , two zeros ] LLeft most 4 bits of PC; offset (26 bits); 00. Compare address of the target instruction and your calculated address froim the binary code of jump instruction. ### 3 Convert the following code into MIPS code and run the code .Use beq instruction. ### if (a ! = b) { c = d – e ; f = b + e ; } else c = a + e ; |
| Lab performed on (date): ___________ Signature: ______________ |  |  |
| Checked by: ________________________ Date: ______________ |  |  |
| Marks Awarded: ____________ |  |  |
| Comments: |  |  |