Lab 02

Resistors color codes and measurement of resistance.

Name: _________________________________________________________________________

Regd. No: _______________________________________________________________________

Components and wires are coded are with colors to identify their value and function.

The colors brown, red, green, blue, and violet are used as tolerance codes on 5-band resistors only. All 5-band resistors use a colored tolerance band. The blank (20%) "band" is only used with the "4-band" code (3 colored bands + a blank "band").

4-Band and 5-Band Resistor Color Chart

Task #1

A resistor colored Yellow-Violet-Orange-Gold

Task #2

A resistor colored Green-Red-Gold-Silver

Task #3

A resistor colored White-Violet-Black

Task#4

A resistor colored Orange-Orange-Black-Brown-Violet

Task #5

A resistor colored Brown-Green-Grey-Silver-Red

Task #6

A resistor colored Blue-Brown-Green-Silver-Blue

Resistor specified with a Resistance value can also be constructed using 4-Bands and 5-Bands of color

A resistor of  1Ω with tolerance of 5% can be designed using the following 4-Bands of color

Brown-black-gold-gold

Same resistance value can also be designed using the following 5-Bands of color

Brown -black -black-silver

Task # 7

Design Resistor with resistance of 5 Ω using 4-Bands of color

Task # 8

Design Resistor with resistance of 6.5 Ω using 4-Bands of color

Task # 9

Design Resistor with resistance of 70 Ω using 5-Bands of color

Task # 10

Design Resistor with resistance of 2.2k Ω using 5-Bands of color

Task # 11

Design Resistor with resistance of 7 MΩ using 4-Bands of color

Task # 12

Design Resistor with resistance of 10 Ω using 5-Bands of color

Task # 13

Design Resistor with resistance of 470 kΩ using 4-Bands of color

| Resistor | Color 1 | Color 2 | Color 3 | Color4 | Coded Resistance | Tolerance | Measured resistance | reading error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |

Conclusion

Summarize what you have learned today (not what you have done).

Checked By:

----------------------------